# WiFi Programmable DC Load

# Two issues with initial run of main PCB (SC 04108221)

## Ground via too close to track

A ground via has been paced too close to the SDA track. The via is across from pin 14 on the CONTROL connector and just above the DAC

2

1

The issue can be corrected, as in the photo, by carefully disconnecting the top of the via from the track and ground plane with a small (2mm) drill bit.

## Regulator orientation is incorrect and heatsink may be required

The 7805 regulator pin markings are reversed on the silkscreen overlay. Input Pin 1 should go to the right, connecting to the 12V input terminals.

The prototype used a VXO7805 switching regulator and no heatsink was specified when the parts list was amended to a 7805 regulator. A small flag heatsink may be required. A ‘touch test” will indicate whether a heatsink is required.